# ABAP 範例程式：Arithmetic Calculations

這個範例程式說明 ABAP 中數字類型數據的計算方式。

程式碼：

1: REPORT demo_data_calculate .

2:

3: * / DIV MOD

4:

5: DATA: pack TYPE p DECIMALS 4,

6:       n TYPE f VALUE '+5.2',

7:       m TYPE f VALUE '+1.1'.

8:

9: pack = n / m.

10: WRITE pack.

11:

12: pack = n DIV m.

13: WRITE  / pack.

14:

15: pack = n MOD m.

16: WRITE / pack.

17:

18: ULINE.

19:

20: * MULTIPLY-CORRESPONDING

21:

22: DATA: BEGIN OF rate,

23:          usa TYPE f VALUE '0.6667',

24:          frg TYPE f VALUE '1.0',

25:          aut TYPE f VALUE '7.0',

26:       END OF rate.

27:

28: DATA: BEGIN OF money,

29:          usa TYPE i VALUE 100,

30:          frg TYPE i VALUE 200,

31:          aut TYPE i VALUE 300,

32:       END OF money.

33:

34: MULTIPLY-CORRESPONDING money BY rate.

35:

36: WRITE / money-usa.

37: WRITE / money-frg.

38: WRITE / money-aut.

39:

40: ULINE.

41:

42: * ADD ...

43:

44: DATA: BEGIN OF series,

45:          n1 TYPE i VALUE 10,

46:          n2 TYPE i VALUE 20,

47:          n3 TYPE i VALUE 30,

48:          n4 TYPE i VALUE 40,

49:          n5 TYPE i VALUE 50,

50:          n6 TYPE i VALUE 60,

51:       END OF series.

52:

53: DATA sum TYPE i.

54:

55: ADD series-n1 THEN series-n2 UNTIL series-n5 GIVING sum.

56: WRITE sum.

57:

58: ADD series-n2 THEN series-n3 UNTIL series-n6 TO sum.

59: WRITE / sum.

第5~16行 試範了  /、DIV、MOD 這三種除法和取餘數的運算。

第20~38行 試範了 MULTIPLY-CORRESPONDING 語句的用法，它會把兩個結構中的同名字段相乘。這種用法現在已經廢棄。

第42~59行 試範了 ADD 語句的特殊用法，可以按照相同間隔累加多個字段的值。這種用法現在也已經廢棄。

要處理數值 數據對象並 將結果值賦 給數據對象 ，可以用 COMPUTE 語句或賦值 運算符（＝ ）。

COMPUTE 語句語法如 下所示：

語法

COMPUTE <n> = <expression>.

關鍵字 COMPUTE 可選。換句 話說，該語 句也可以寫 成：

語法

<n> = <expression>.

兩條語句效 果等同。

在 <expression> 中指定的數 學運算結果 賦給字段 <n>。賦 值運算符（ ＝）的作用 如基本賦值操 作 (頁 38) 中所述。 如果運算結 果與 <n> 的數據類型 不同，則系 統自動進行 類型轉換（ 參見類型轉換 (頁 20) ）。

在數學表達 式中，可以 按任何排列 組合運算並 用括號進行 指定。

求值順序是 ：

- 括號 中的表達式

- 函數

- ** （求冪）

- *、 /、MOD、 DIV （乘法、除 法）

- +、 – （加法、減 法）