鼎基先進

10G Switch 設定

(config)

Login 10G Switch

ssh 192.168.10.63 & 192.168.10.64 User:admin Password:admin DZSAPSW1#

Set Stack(兩台Switch做堆疊，使用te1/1/25、te1/1/26作為堆疊port)

| 先登入第一台10g Switch作設定 S1# configure terminal S1(config)# spanning-tree mode rstp S1(config)# spanning-tree rstp priority 4096 S1(config)# interface range e1/1/25-1/1/26 S1(conf-range-eth1/1/25-1/1/26)# no switchport S1(conf-range-eth1/1/25-1/1/26)# no shutdown S1(conf-range-eth1/1/25-1/1/26)# exit S1 (config)# interface mgmt 1/1/1 S1 (conf-if-ma-1/1/1)# no ip address dhcp S1 (conf-if-ma-1/1/1)# ip address 192.168.10.63/24 S1 (conf-if-ma-1/1/1)# no shutdown S1 (conf-if-ma-1/1/1)#exit S1(config)# vlt-domain 1 S1(conf-vlt-1)# discovery-interface ethernet 1/1/25 S1(conf-vlt-1)# discovery-interface ethernet 1/1/26 S1(conf-vlt-1)# vlt-mac 0a:0b:0c:dd:dd:ff S1(conf-vlt-1)# backup destination 192.168.10.64 S1(conf-vlt-1)# delay-restore 10 S1(conf-vlt-1)# peer-routing S1(conf-vlt-1)#exit S1(config)# interface port-channel 1 S1(conf-if-po-1)# description "Connect to S2 switch" S1(conf-if-po-1)# vlt-port-channel 1 *S1(conf-if-po-1)# switchport mode trunk *S1(conf-if-po-1)# switchport trunk allowed vlan 10 S1(conf-if-po-1)# no shutdown S1(conf-if-po-1)# exit *S1(config)# interface ethernet e1/1/29 *S1(conf-range-eth1/1/29)# channel-group 1 mode active *S1(conf-range-eth1/1/29)# no shutdown *S1(conf-range-eth1/1/29)# exit | 登入第二台10g Switch作設定 S2# configure terminal S2(config)# spanning-tree mode rstp S2(config)# spanning-tree rstp priority 4096 S2(config)# interface range e1/1/25-1/1/26 S2(conf-range-eth1/1/25-1/1/26)# no switchport S2(conf-range-eth1/1/25-1/1/26)# no shutdown S2(conf-range-eth1/1/25-1/1/26)# exit S2 (config)# interface mgmt 1/1/1 S2 (conf-if-ma-1/1/1)# no ip address dhcp S2 (conf-if-ma-1/1/1)# ip address 192.168.10.63/24 S2 (conf-if-ma-1/1/1)# no shutdown S2 (conf-if-ma-1/1/1)#exit S2(config)# vlt-domain 1 S2(conf-vlt-1)# discovery-interface ethernet 1/1/25 S2(conf-vlt-1)# discovery-interface ethernet 1/1/26 S2(conf-vlt-1)# vlt-mac 0a:0b:0c:dd:dd:ff S2(conf-vlt-1)# backup destination 192.168.10.63 S2(conf-vlt-1)# delay-restore 10 S2(conf-vlt-1)# peer-routing S2(conf-vlt-1)#exit S2(config)# interface port-channel 1 S2(conf-if-po-1)# description "Connect to S1 switch" S2(conf-if-po-1)# vlt-port-channel 1 *S2(conf-if-po-1)# switchport mode trunk *S2(conf-if-po-1)# switchport trunk allowed vlan 10 S2(conf-if-po-1)# no shutdown S2(conf-if-po-1)# exit *S2(config)# interface ethernet e1/1/29 *S2(conf-range-eth1/1/29)# channel-group 1 mode active *S2(conf-range-eth1/1/29)# no shutdown *S2(conf-range-eth1/1/29)# exit |
| --- | --- |

Show running-config(查看執行設定檔)

| DZSAPSW1# show running-configuration 查看使用中設定檔 | DZSAPSW2# show running-configuration 查看使用中設定檔 |
| --- | --- |
| DZSAPSW1# show running-configuration ! Version 10.5.0.4 ! Last configuration change at Jan 13 15:25:26 2021 ! ip vrf default ! interface breakout 1/1/25 map 100g-1x interface breakout 1/1/26 map 100g-1x hostname DZSAPSW1 iscsi enable iscsi target port 860 iscsi target port 3260 system-user linuxadmin password $6$5DdOHYg5$JCE1vMSmkQOrbh31U74PIPv7lyOgRmba1IxhkYibppMXs1KM4Y.gbTPcxyMP/PHUkMc5rdk/ZLv9Sfv3ALtB61 spanning-tree mode rstp spanning-tree rstp priority 4096 username admin password $6$q9QBeYjZ$jfxzVqGhkxX3smxJSH9DDz7/3OJc6m5wjF8nnLD7/VKx8SloIhp4NoGZs0I/UNwh8WVuxwfd9q4pWIgNs5BKH. role sysadmin priv-lvl 15 aaa authentication login default local aaa authentication login console local ! class-map type application class-iscsi ! policy-map type application policy-iscsi ! interface vlan1 no shutdown ! interface port-channel1 description "Connection to S3 Switch" no shutdown switchport access vlan 1 vlt-port-channel 1 ! interface mgmt1/1/1 no shutdown no ip address dhcp ip address 192.168.10.63/24 ! interface ethernet1/1/1 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/2 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/3 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/4 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/5 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/6 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/7 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/8 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/9 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/10 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/11 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/12 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/13 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/14 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/15 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/16 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/17 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/18 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/19 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/20 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/21 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/22 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/23 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/24 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/25 no shutdown no switchport flowcontrol receive on ! interface ethernet1/1/26 no shutdown no switchport flowcontrol receive on ! interface ethernet1/1/27 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/28 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/29 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/30 no shutdown switchport access vlan 1 flowcontrol receive on ! vlt-domain 1 backup destination 192.168.10.64 delay-restore 10 discovery-interface ethernet1/1/25-1/1/26 peer-routing vlt-mac 0a:0b:0c:dd:dd:ff ! snmp-server contact "Contact Support" ! telemetry DZSAPSW1# | DZSAPSW2# show running-configuration ! Version 10.5.0.4 ! Last configuration change at Jan 13 15:17:53 2021 ! ip vrf default ! interface breakout 1/1/25 map 100g-1x interface breakout 1/1/26 map 100g-1x hostname DZSAPSW2 iscsi enable iscsi target port 860 iscsi target port 3260 system-user linuxadmin password $6$5DdOHYg5$JCE1vMSmkQOrbh31U74PIPv7lyOgRmba1IxhkYibppMXs1KM4Y.gbTPcxyMP/PHUkMc5rdk/ZLv9Sfv3ALtB61 spanning-tree mode rstp spanning-tree rstp priority 8192 username admin password $6$q9QBeYjZ$jfxzVqGhkxX3smxJSH9DDz7/3OJc6m5wjF8nnLD7/VKx8SloIhp4NoGZs0I/UNwh8WVuxwfd9q4pWIgNs5BKH. role sysadmin priv-lvl 15 aaa authentication login default local aaa authentication login console local ! class-map type application class-iscsi ! policy-map type application policy-iscsi ! interface vlan1 no shutdown ! interface port-channel1 description "Connection to S1 Switch" no shutdown switchport access vlan 1 vlt-port-channel 1 ! interface mgmt1/1/1 no shutdown no ip address dhcp ip address 192.168.10.64/24 ! interface ethernet1/1/1 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/2 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/3 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/4 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/5 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/6 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/7 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/8 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/9 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/10 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/11 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/12 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/13 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/14 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/15 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/16 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/17 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/18 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/19 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/20 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/21 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/22 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/23 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/24 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/25 no shutdown no switchport flowcontrol receive on ! interface ethernet1/1/26 no shutdown no switchport flowcontrol receive on ! interface ethernet1/1/27 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/28 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/29 no shutdown switchport access vlan 1 flowcontrol receive on ! interface ethernet1/1/30 no shutdown switchport access vlan 1 flowcontrol receive on ! vlt-domain 1 backup destination 192.168.10.63 delay-restore 10 discovery-interface ethernet1/1/25-1/1/26 peer-routing vlt-mac 0a:0b:0c:dd:dd:ff ! snmp-server contact "Contact Support" ! telemetry DZSAPSW2# |